Slide 1 – Title Good morning, everyone. Please sit down and take your notebooks. Today we are going to talk about Properties of Materials — elastic deformation, plastic deformation, alloys, heat treatment, and mechanical properties. These topics are very important for your future work with medical equipment in hospitals.

Slide 2 – Main question Let’s start with the main question of today’s lecture. Look at these two wires. I apply the same force to both. One wire returns to its straight shape. The other wire stays bent and does not return. Why does this happen? Same force — completely different result. Today we will understand exactly why materials behave differently.

Slide 3 – Atomic Crystal Structure of Metals First, we need to understand how metals are built inside. Metals have a very regular structure called a crystal lattice. The atoms are arranged in a perfect, orderly pattern, like soldiers standing in straight lines. This regular crystal lattice gives metals their special properties. Different metals have different crystal structures, for example FCC or BCC. Look at the picture — you can see this perfect order of atoms.

Slide 4 – Metallic Bonding (Electron Sea) Inside the metal, there are positive metal ions fixed in their positions. Around them is a “sea” of free negative electrons. These electrons move freely and hold the positive ions together with electrostatic forces. When we stretch the metal, the distance between ions increases. When we compress it, the distance decreases. The electrostatic forces try to bring the ions back to their normal positions. This is the basic reason for elasticity in metals.

Slide 5 – Elastic Deformation When we apply a small force, the atoms move only a little. They stretch or compress, but they stay in their original positions in the lattice. When we remove the force, the atoms return exactly to their original places. The material returns to its original shape. This is called elastic deformation. Examples are springs, rubber bands, and thin steel wires. Look at the picture — you see the atoms stretching and then returning back.

Slide 6 – Plastic Deformation When we apply a very large force, something different happens. This is plastic deformation. The atoms start to slide past each other along special slip planes. They change their positions permanently. When the force is removed, the material does not return to its original shape — it stays changed. Examples: bent copper wire, aluminum foil, or a bent spoon. Look at the picture — you can see atoms sliding along the slip planes.

Slide 7 – Comparison: Elastic vs Plastic Deformation Let’s compare them. In elastic deformation: small force, atoms stretch and return, shape comes back. In plastic deformation: large force, atoms slide permanently, shape stays changed. This difference is very important. Please look at the table on the slide — it clearly shows the main differences.

Slide 8 – Alloys Pure metals are often too soft or too weak. That is why we make alloys — mixtures of two or more metals. Alloys are usually stronger and harder than pure metals. There are two main types: substitution solid solution and interstitial solid solution. Examples: bronze is copper plus tin, steel is iron plus carbon. Look at the pictures — you can see how atoms are arranged in alloys.

Slide 9 – Why alloys are stronger Why are alloys stronger? In pure metal, all atoms are the same size and can slide easily. In an alloy, different sized atoms disturb the perfect lattice. It becomes much more difficult for atoms to slide. That is why alloys are stronger and harder. This is very important for tools and medical instruments.

Slide 10 – Heat Treatment and Quenching Heat treatment means controlled heating and cooling of metal. Quenching is when we heat the metal very hot and then cool it very quickly in water or oil. After quenching, the metal becomes much harder and stronger, but it can also become more brittle. Look at the picture — you see hot metal going into cold water.

Slide 11 – Japanese Katana Japanese sword makers were masters of materials science. The katana has a multi-layer structure. The edge has high carbon and is quenched — it is very hard and sharp. The core is softer and more flexible — it gives toughness so the sword does not break. Look at the pictures — you can see the layered structure and the grain of the metal.

Slide 12 – Mechanical Properties of Materials Now we will talk about the most important mechanical properties. We need to know them to choose the right material for medical tools.

Slide 13 – Hardness Hardness is how difficult it is to scratch or make a dent in the material. We measure hardness with special machines. The most common methods are Rockwell, Brinell, and Vickers tests. In these tests, we press a hard ball or a diamond tip into the surface of the material and measure how deep the indentation is. The harder the material, the smaller the indentation. Diamond is the hardest natural material in the world. In hospitals, surgical scalpels, knives, and other cutting tools must have very high hardness so they stay sharp for a long time and can cut cleanly. If the material is not hard enough, the tool will become dull very quickly. Look at the pictures — you see how we measure hardness.

Slide 14 – Strength and Toughness Strength is how much load the material can carry without breaking. We measure strength with a tensile testing machine. We pull the sample until it breaks. During this test, at some point the sample becomes thinner in one place. This is called necking. After necking, the material quickly breaks.

Toughness is how much energy the material can absorb before it breaks. A good surgical scalpel needs both high strength and good toughness. It must be strong enough to cut through tissue, but not too brittle so it doesn’t break during surgery. Look at the pictures — you can see the tensile test and how necking happens before the final fracture.

Вопрос студентам (задаёшь ты):

"Here we have two materials.

Material 1: Low-carbon mild steel. It is relatively soft, bends easily, and has good ductility (plasticity).

Material 2: White cast iron. It is very hard, but brittle and almost does not bend.

Question: Which material do you think will better resist breaking when we pull it (tensile force)? Which one can withstand a larger force before it finally breaks?

Low-carbon mild steel or white cast iron? Think about it and explain your answer."

(Дай им 30–60 секунд на размышление и несколько ответов.)

Твой ответ студентам (после их ответов):

"Many of you probably chose white cast iron because it is harder. This is a common mistake.

Actually, low-carbon mild steel will resist breaking much better under tensile force than white cast iron.

Why?

White cast iron is very hard, but extremely brittle. It has almost no plastic deformation. When the force reaches a certain level, it suddenly cracks and breaks without any warning.

Low-carbon mild steel is softer, but it has good ductility. It can stretch noticeably, form a neck (necking), and only after that it breaks. Because of this plastic deformation, it can absorb much more energy and withstand a significantly higher tensile force before final fracture.

However, the best solution in real engineering is usually not the extremes, but the golden mean — a material that has enough hardness and strength, but also moderate ductility and toughness.

Such a balanced material can carry high loads, gives some warning before breaking, and does not lose its shape too early. This is why balanced materials are most often used in medical tools and hospital equipment."

Slide 15 – Summary Today we learned many important things:

- The difference between elastic and plastic deformation

- How atoms behave inside metals

- What alloys are and why they are stronger

- Basics of heat treatment and quenching

- The main mechanical properties: hardness, strength, elasticity, plasticity, toughness, and brittleness

All this knowledge will help you choose the right materials for medical tools and hospital equipment. In the next lecture we will talk about tools and material processing. Thank you very much for your attention. Do you have any questions?